Framing in Data Link Layer

Definition

Framing is the process of dividing a continuous stream of bits into frames, which are the basic units of data transmission in the Data Link Layer (DLL) of the OSI model.

A frame contains:

- User data (payload)

- Source and destination addresses

- Error detection information (CRC/Checksum)

- Control information

Why is Framing Needed?

- Identifies the start and end of each frame.

- Provides source and destination addressing.

- Supports error detection and correction.

- Ensures reliable and synchronized communication.

- Makes data transmission organized and efficient.

Note: Framing is completely handled by the Data Link Layer and is transparent to the user.

Types of Framing

1. Fixed-Size Framing

Definition

All frames have the same predefined size.

Characteristics

- No start or end delimiters are required.

- Frame boundary is identified by its fixed length.

Advantages

- Simple implementation.

- Easy synchronization.

- Faster processing.

Disadvantages

- Causes internal fragmentation when the data is smaller than the frame size.

- Wastes bandwidth.

Solution

Padding is added to fill the unused space.

2. Variable-Size Framing

Definition

Frames can have different sizes depending on the amount of data.

Frame boundaries are identified using:

- Length Field

- End Delimiter (Flag)

(a) Length Field Method

The frame contains a length field that specifies the size of the frame.

Example

| Header | Length = 50 Bytes | Data | CRC |

Advantage

- Easy to determine frame size.

Disadvantage

- If the length field is corrupted, the receiver cannot determine the frame boundary correctly.

(b) End Delimiter Method

A special bit or byte pattern marks the end of the frame.

Example:

Flag = 01111110

Problem:
The same pattern may appear inside the actual data.

Solution:
Use Stuffing Techniques.

Example Without End Delimiter

Assume two frames are transmitted.

Frame 1

11001010

Frame 2

10101111

Without any delimiter, the transmitted data becomes:

1100101010101111

The receiver receives:

1100101010101111

It cannot determine:

- Where Frame 1 ends.

- Where Frame 2 starts.

Example With End Delimiter

Suppose the End Delimiter (ED) is

1111

The sender transmits:

Frame 1 : 11001010 1111

Frame 2 : 10101111 1111

The transmitted bit stream becomes:

11001010 1111 10101111 1111

At the Receiver

The receiver reads bits continuously.

11001010 1111

When it encounters 1111, it understands:

Frame 1 has ended.

Then it continues reading:

10101111 1111

Again, 1111 indicates:

Frame 2 has ended.

Thus, the receiver correctly separates the frames.

Problem with End Delimiter

Suppose the delimiter is

ED = 1111

Now the actual data itself contains 1111.

Example:

Data = 101111001

If the sender simply transmits

1011110011111

the receiver sees:

1011110011111

When it reads the first 1111 inside the data, it mistakenly assumes:

The frame has ended here!

This is incorrect because those bits belong to the data.

Solution – Bit Stuffing

To avoid confusion, whenever the delimiter pattern appears in the data, the sender inserts an extra bit.

For example,

Delimiter:

1111

Original data:

101111001

After bit stuffing (illustrative):

1011101001

Now the delimiter pattern no longer appears inside the data.

At the receiver:

- The extra stuffed bit is removed (de-stuffing).

- The original data is restored correctly.

- The receiver recognizes the actual 1111 only when it appears as the End Delimiter.

Stuffing Techniques

1. Character (Byte) Stuffing

Definition

Used in character-oriented protocols.

Whenever a special delimiter character appears in the data, an escape character is inserted before it.

Example

Suppose:

Delimiter = $

Escape = \

Original Data

ABC$XYZ

After Byte Stuffing

ABC\$XYZ

Advantages

- Simple to understand.

- Suitable for text-based communication.

Disadvantages

- Extra processing overhead.

- Rarely used in modern high-speed networks.

2. Bit Stuffing

Definition

Used in bit-oriented protocols.

Whenever the delimiter pattern appears in the data, an extra 0 bit is inserted so that the receiver does not confuse it with the frame delimiter.

Example

Delimiter

01111110

Data

01111110

After Bit Stuffing

011111010

The receiver removes the inserted 0 during de-stuffing.

Advantages

- Efficient for binary communication.

- Widely used in modern protocols (e.g., HDLC).

Problems in Framing

- Frame Boundary Detection

  - Receiver must correctly identify the start and end of each frame.

- Synchronization

  - Sender and receiver must remain synchronized during transmission.

- Error Handling

  - Noise may corrupt the frame or delimiters.

  - CRC and checksums are used for error detection.

- Overhead

  - Headers, trailers, and stuffing increase frame size.

- Efficiency

  - Balance between payload size and control information.

- Compatibility

  - Different framing methods may lead to interoperability issues.

Comparison: Fixed vs Variable Framing

| Feature | Fixed-Size Framing | Variable-Size Framing |
| --- | --- | --- |
| Frame Size | Constant | Varies |
| Boundary Detection | Frame length | Length field or delimiters |
| Complexity | Simple | More complex |
| Efficiency | Lower (padding required) | Higher |
| Bandwidth Usage | Less efficient | More efficient |
| Fragmentation | Internal fragmentation | Minimal |

Advantages of Framing

- Reliable data transmission.

- Easy error detection.

- Proper synchronization.

- Efficient flow control.

- Organized communication.

- Supports addressing and control information.